# README – CSC and Kou indices

This folder contains the Python code and input data used to compute the Clyne-Davies crack susceptibility coefficient (CSC), the Kou cracking susceptibility index (Kou CSI), and the associated liquidus temperature (TL), solidus temperature (TS), and freezing range ΔTfor the W-Ti-V, W-Ti-Zr, and W-V-Zr ternary systems.

## Files

- CSC_Kou.ipynb – Python code used for the calculations

- WTiV/ – folder containing Thermo-Calc/Scheil CSV outputs for W-Ti-V

- WTiZr/ – folder containing Thermo-Calc/Scheil CSV outputs for W-Ti-Zr

- WVZr/ – folder containing Thermo-Calc/Scheil CSV outputs for W-V-Zr

- CSC_all_systems.xlsx – output file generated by the code

## What the code does

For each Thermo-Calc/Scheil CSV file, the code:

- reads the liquid fraction, cumulative heat, and temperature data,

- computes cumulative heat at fL=0.60, 0.10, and 0.01,

- computes the Clyne-Davies CSC:

CSC=tvtR
with

tR=(ΔQ0.10-0.60)2,tv=(ΔQ0.01-0.60)2-(ΔQ0.10-0.60)2

- computes the Kou cracking susceptibility index from the terminal solidification path,

- extracts:

  - TL [K] as the temperature at maximum liquid fraction,

  - TS [K] as the temperature at minimum liquid fraction,

  - ΔT=TL-TS

## Output

The output workbook CSC_all_systems.xlsx contains one sheet for each ternary system: WTiV, WTiZr, and WVZr.

## Notes

- The same workflow was used for all three ternary systems.

- Composition id are sequential and follow compositions given in Solidification/spreading index file (useful information for plotting).